Section 19 LOTS, YARDS AND DRIVEWAYS

|  | Agricultural Protection District (A-1) | Agriculture District (A-2) | Residential District (R-1) | Rural Business District (B-2) | Subdivisions | ##### Public Park ##### District ##### (P-1) |
| --- | --- | --- | --- | --- | --- | --- |
| Residential density maximum | One dwelling per 160 acres and one dwelling per quarter section | One dwelling unit per 20 acres and 12 dwellings per section and one dwelling per quarter, quarter section | One dwelling unit per five acres and 12 dwellings per section and one dwelling per quarter, quarter section | Not Applicable |  | Not Applicable |
| Minimum lot size | 160 acres | 20 acres, or 5 acres where *CPI does not Exceed 50% | 5 acres | 5,000 square feet | 2 acres | 30 acres |
| Minimum lot width | 300 feet | 300 feet | 200 feet |  | 100 feet | 1,000 feet |
| Minimum frontage on public road | 100 feet | 100 feet | 100 feet |  | 100 feet | 500 feet |
| Setbacks Minimum front yard Minimum side yard Minimum rear yard Livestock Use Imposes 100 feet of setback on all sides | 100 feet 30 feet 50 feet | 100 feet 30 feet 50 feet | 100 feet 15 feet 15 feet | 45 feet 0 feet 20 feet* * where alleys exist, the rear yard may include one-half the width of the alley | 58 feet 15 feet 15 feet | 100 feet 100 feet* 100 feet* |
| Driveway setback from side or rear lot line | 5 feet | 5 feet | 5 feet | Not Applicable | 5 feet | 5 feet |
| Minimum floor area per dwelling unit | 900 square feet | 900 square feet | 900 square feet | Not applicable | 900 square feet | Not Applicable |
| Maximum structure Height | 35 feet | 35 feet | 35 feet | 35 feet | 35 feet | 35 feet |

* Facilities specifically oriented to waterways may be allowed a reduced setback.

*CPI is the Crop Productivity Index